- SECTION 01 35 13

- SPECIAL PROCEDURES FOR WORKING IN PARK PROPERTY

- (This spec should only be included when working within City of Cincinnati Park Property and may need to be modified per project specific conditions – Consult with MSD project manager and/or Parks Department)

- PART 1 – GENERAL

SECTION INCLUDES

Protection of Trees

Protective Fencing

Accessways

Trail Signage

RELATED SECTIONS

- Measurement and Payment 01 20 00

PROTECTION OF TREES

- The CONTRACTOR shall take precaution to avoid any unnecessary damage to trees on either private or Park Board property. Such measures shall include erecting barricades, limiting access to the project to designated ways, trimming trees to avoid aerial construction damage, and installing pipe and other work by hand excavation or tunneling methods. Such trees shall not be endangered by stockpiling excavated material or storing equipment within the drip line of the tree canopy.

Branches which over hang the Construction Limits and which interfere with the operation of equipment shall be removed following ANSI A300 Arborist standards. A copy of the standards are available for reading at the Park operations office located at 3215 Reading Road, Cincinnati, 45229

If any digging is done with 15 feet of any tee designated for protection, and a root greater than 4 inches must be severed, CONTRACTOR shall contact the Park Board at 513.861.9070 for a site inspection. If the tree must be removed, CONTRACTOR shall give the city at least 3 working days so that the CITY can schedule the tree and stump removal.  Tree and stump removal is paid directly by the city to the tree service contractor under contract to the CITY and will not be measured for payment to the CONTRACTOR. Meeting these requirements are considered incidental to the contract and will not be measured for payment or a reason to extend construction time.

The CONTRACTOR shall work with the CITY to make field adjustments as necessary to work in order to promote long- term health and viability of the trees.

Any tree(s) designated for protection that is damaged by the CONTRACTOR beyond saving shall be removed by the CONTRACTOR at no additional cost to the CITY. The tree value(s) shall be determined using the size, condition, species, and location value for the tree per section 743 of the Cincinnati Municipal Code.  The CONTRACTOR shall make compensation directly to the Park Board at 3215 Reading, Cincinnati, OH 452293.

Bat trees that must be removed shall be removed prior to mobilizing equipment and starting construction.

PROTECTIVE FENCING

- The CONTRACTOR shall take extra measures to protect trees and vegetation designated to be preserved. The CITY shall field designate, and the CONTRACTOR shall install orange construction fence along the Construction Limits, protected areas, haul roads.  Haul roads and  accessways shall be no more than 12 feet wide, with construction fencing placed along each side.

ACCESSWAYS

- Unless authorized by the CITY, the CONTRACTOR shall not enter, disturb or alter any area outside the Construction Limits, within a designated prohibited area surrounded by construction fencing; nor shall the CONTRACTOR allow runoff from disturbed areas of the project to enter said area.

TRAIL SIGNAGE

The CONTRACTOR shall provide signage at locations designated on the Drawings to indicate trails that are temporarily closed during construction.  Signs to be placed at nearest exterior trailheads and at trail intersections near the Construction Limits.

- PART 2 – PRODUCTS

  - FORESTED WETLAND SEED MIX

- Grasses and Sedges

- Oz/acre

- 2    Carex frankii (Frank’s Sedge)

- 1    Carex granularis (Meadow Sedge)

- 2    Carex grayi (Burr Sedge)

- 2    Carex lupulina (Common Hop Sedge)

- 1    Carex muskingumensis (Palm Sedge)

- 1    Carex normalis (Speading Oval  Sedge)

- 1    Carex tribuloides (Pointed Oval Sedge)

- 2    Carex vulpinoidea (Fox Sedge)

- 16    Elymus riparius (Riverbank Wild Rye)

- 64    Elymus virginicus (Virginia Wild Rye)

- 2    Glyceria striata (Fowl Manna Grass)

- 2    Hystrix patula (Bottlebrush Grass)

- 96	oz/acre Grasses and Sedges seed

- Forbs

- 2    Actinomeris alternifolia (Wingsterm)

- 2    Aster lateriflorus (Side- Flowering Aster)

- 1    Aster puniceus (Swamp Aster)

- 2    Aster simplex (Panicled Aster)

- 0.5     Blephilia hirsuta (Hairy Wood Mint)

- 2      Helenium autumnale ( Autum Sneezeweed)

- 2      Heliopsis helianthoides (False Sunflower)

- 0.5     Lobelia siphilitica (Great Blue Lobelia)

- 1.5     Lycopus americanus (Water Horehound)

- 0.5     Mimulus ringens (Monkey Flower)

- 1     Penstemon calysosus (Smooth Penstemon)

- 2     Rudbeckia laciniata (Green-Headed Coneflower)

- 3     Silphium perfoliatum (Cupplant)

- 1.5     Solidago gigantean (Late Goldenrod)

- 0.5     Veronicastrum virgincum (Culver’s Root)

- 2     Zizia aurea (Golden Alexanders)

- 24	oz/acre  Forbes seed

- Total Seed per acres, Forbes, Grasses and Sedges at 120 oz/acre

- PART 3 - EXECUTION

  - CLEARING AND GRUBBING

- After construction fencing has been installed, the CONTRACTOR shall walk the project with CITY personnel to locate trees that to be saved and trees to be removed. All trees will be clearly numbered and marked with paint to indicate status.

- Trees marked with “LOG” shall have the brush, tops, and limbs removed and the trunk section shall be cut into 10’ lengths and hauled to location specified by the CITY. CONTRACTOR shall cause minimal damages to log sections while dropping and transporting.

  - RESTORATION

- The CONTRACTOR shall restore unpaved areas within park property with forested wetland seed mix.  Seed must have been collected within 100 miles of Cincinnati and source must be approved by the Park Board prior to installation.  Seed mix can be obtained from Spence Restoration.  Seeding shall occur between September 15th and November 1st.  Seeded areas shall be mulched per ODOT specifications.

- Temporary restoration on Park Property.  If necessary to control erosion or to provide temporary groundcover until permanent restoration can be achieved, CONTRACTOR shall seed with a 100% mixture of Annual Ryegrass per manufacturer’s specified rate.

- CONTRACTOR shall restore all trails used for access or haul roads to pre-existing conditions at no cost to the CITY.

- Restoration areas to be planted in Forbs or planted in Grasses or Sedges are as noted on plans.  If restoration areas are not noted on the plans, Forbs shall be used.

  - PAYMENT

- All costs associated with Special Procedures for Working in Park Property shall be included with various contract items.