SECTION 01 45 16

CONTRACTOR QUALITY CONTROL

[Use this Specification if complexity and/or scope warrant use, typically for WWIP projects or large plant jobs… consult with MSD Project Manager]

PART 1 - GENERAL

1.01 SECTION INCLUDES:

A.  Related Sections

B.  Definable Features of Work

C.  Three Phases of Quality Control Inspection

D.  Quality Control Organization

E.  Quality Testing and Inspection

F.  Quality Reporting

1.02 RELATED SECTIONS

A.  Submittals (01 33 00)

B.  Request for Substitution (Material and Installation Method)

C.  Request for Information

D.  Commissioning (01 75 00 – 01 75 06)

E.  Testing Laboratory Services (01 45 29)

1.03 DEFINABLE FEATURES OF WORK (DFOW)

A.  A DFOW results in a physical product, can be identified by a unique trade or a set of coordinated trades, and requires a distinct set of quality control activities.

B. The CONTRACTOR is required to prepare a list of DFOW for the project.

C.  DFOW Selection Process:

1.  Begin developing the DFOW list by looking at each technical specification section.

2.  Add to or replace general category task with tasks that meet the following criteria:

a. It requires quality controls such as inspections

b. It has operations, work crews, or disciplines unique to the task

c. It is a critical path activity on the schedule

d. It is monitored by inspection personnel

D.  For Construction Projects which require Commissioning, at a minimum DFOWs shall align with Specification Section 01 75 00 through 01 75 06.

1.04 THREE PHASES OF QUALITY CONTROL INSPECTION

A.  GENERAL

1.  Contractor Quality Control (CQC) is the means by which the CONTRACTOR ensures that the construction, including that of subcontractors and suppliers, complies with the requirements of the contract.

2.  The core of the CQC Plan is the implementation of Three Phases of Quality Control Inspection to be conducted by the Quality Control Manager and designated Quality Control Specialists for each DFOW.

B. THREE PHASES OF QUALITY CONTROL INSPECTION

1. PREPARATORY

a. This phase shall be performed PRIOR TO BEGINNING work on each DFOW, after all required documents and materials are approved/accepted, and after copies are at the work site.

b. The QUALITY CONTROL MANAGER shall conduct a Preparatory Phase Meeting attended by the Construction Superintendent, other QC   Specialists, Foreman (Subcontractor), City Construction Management, City Safety, and other pertinent CITY representatives.

c. The following items are discussed at the Preparatory Phase Meeting as prompted by the Preparatory Phase Meeting Checklist:

1.  Review of each paragraph of applicable specifications, reference codes, and standards.

2.  Review of Contract Drawings

3.  Verify submittal(s) approval

3.  Physical examination of material and equipment based on shop drawings and approved submittals.

4.  Discussion of procedures for construction:

a. Review of the detailed sequence of the execution of the work.

b. Review testing plan

5.  Examination of the work area to assure that all required preliminary work has been completed and is in compliance with the contract.

6.  Review of applicable Safety requirements

d. Meeting Minutes shall be recorded and attached to the QC Daily Report.

e. Repeat Preparatory meeting and initial inspection if:

1. Deficiencies persist

2. Work on a DFOW resumes after a long pause

3. Large turnover in construction crew

2. INITIAL

a. This phase shall be performed at the beginning of each DFOW and repeated for each new workman or crews assigned to work on site or when level of workmanship is not being met.

b. The QUALITY CONTROL MANAGER shall conduct an Initial Phase Meeting attended by the Construction Superintendent, other QC Specialists, Foreman (Subcontractor), City Construction Management, City Safety, and other pertinent CITY representatives.

c. Meeting Minutes shall be recorded and attached to the QC Daily Report.

d. The following items are discussed at the Initial Phase meeting as prompted by the Initial Phase Checklist:

1. Review the minutes of the Preparatory Meeting and verify that the work complies with the design documents (i.e. submittals, specifications and/or shop drawings)

2. Check all Preliminary work is done

3. Resolve all differences and deficiencies

4. Establish quality required to ensure full contract compliance

5. Ensure testing is performed

6. Review approved Safety Plan – check for enforcement

7. Provide documentation of the previous inspection of the work area.

8. Re-examine the work area for compliance.

3. FOLLOW-UP

a. Daily checks performed by Contractor Quality Control personnel during progress of the DFOW.

b. Performed to assure continuing compliance with contract requirements, including control testing, until completion of the particular work activity.

- Ensure testing reports are submitted

- Ensure reworks are completed

c. CQC personnel continually refer back to standards set in Preparatory Meeting and Initial Inspection.

d. Control safety and workmanship of testing until completion of DFOW

e. Final follow-up checks shall be conducted, and all noted deficiencies corrected, prior to the start of additional features of work that may be affected by the deficient work.

f. The checks shall be made a matter of record in the Contractor QC Daily Report.

1.05 QUALITY CONTROL ORGANIZATION

- CONTRACTOR’S RESPONSIBILITY

- The CONTRACTOR is responsible for the quality of all materials and Work including the Work of its Subcontractors, vendors and suppliers; and for assuring that the quality of Work specified in the Contract Documents is achieved.

- The CONTRACTOR shall provide evidence/documentation that all items have been submitted, tested, inspected, accepted, etc. and that each step was conducted in the appropriate time frame to control work as it progresses.

1.06 CONTRACTOR QUALITY CONTROL ORGANIZATION

- It is the CITY’s intent that the workforce supplied by the CONTRACTOR, Subcontractors, Suppliers, Manufacturers and Independent Testing Agencies for the Project meets or exceeds certain minimum experience qualifications, and that this experience can be documented based on other previous projects similar in size and scope to the Project.  The CONTRACTOR shall provide evidence to demonstrate compliance with the minimum experience qualifications required herein upon request of the CITY.  Whenever the Contract Documents use the terms listed below, CONTRACTOR shall ensure that the minimum experience qualifications stated below shall be met or exceeded by CONTRACTOR, Subcontractors, and Suppliers.

- Minimum Personnel and Experience Requirements:

1. QUALITY CONTROL MANAGER – Confirm on resume at least 5 years of related experience in quality control inspection on construction projects.

2. QUALITY CONTROL SPECIALISTS – Confirm on resume at least 2 years of related experience under the direction of a Project Manager, Construction Superintendent, or Quality Control Manager.

3. CONSTRUCTION SUPERINTENDENT – Confirm on resume either a degreed graduate of engineering, architecture or construction management with 5 years of related experience or 10 years of relevant experience.

4. PROJECT MANAGER – Confirm on resume either a degreed graduate of engineering, architecture or construction management with 5 years of related experience or 10 years of relevant experience.

5. SITE SAFETY MANAGER – Confirm on resume at least 5 years of related experience.

6. FOREMAN – Confirm on resume at least 5 years of related experience.

7. Installers/Laborers - Confirm on resume at least 2 years of related experience working under the direction of a Project Manager or Construction Superintendent.  Provide copy of current license and/or certification to drive/operate the type of vehicle/equipment which they drive/operate either on-site or off-site.

8. Manufacturers - Provide documentation of having a minimum of 5 years’ experience in manufacture and successful start-up of the specified equipment/design demonstrated by submitting at list of 5 similar projects completed within the last 5 years including project names, addresses, contact names, addresses & telephone numbers of owners and any other type of information specified or requested by the CITY.

9. Independent Testing Agencies - Provide documentation of having a minimum of 5 years’ experience performing the specified independent testing required by the Contract Documents for review and approval by the CITY.  Submit a list of 5 similar projects completed within the last 5 years including project names, addresses, contact names, addresses & telephone numbers of owners and any other type of information specified or requested by the CITY.

C. Quality Control Manager

1. The Quality Control (QC) Manager will be responsible for overseeing overall implementation of the Quality Control Plan and will coordinate all project testing, inspections and reporting matters directly with the Project Manager.  The QC Manager has the authority to intercede directly and stop unsatisfactory work and control further processing, delivery or installation of non-conforming material.

2. Responsibilities include but are not limited to:

a. Preparation, approval and implementation of the CQC Plan

b. Verification of materials as per project plans and specifications

c. Development of means and methods to store and protect materials

d. Preparation of acceptable documentation of CQC activities

e. Maintain documentation of inspection status of materials

f. Maintain documentation for material and administrative approvals

g. Ensure that all materials and construction are in accordance with the requirements for the completeness, accuracy and constructability in accordance with applicable building codes

h. Carry out and participate in weekly progress and QC meetings

Maintain documentation of inspection of work executed by subcontractors

i. Stopping Work

D. The CONTRACTOR shall maintain the CQC staff at full strength at all times.  When it is necessary to make changes to the CQC staff, the CONTRACTOR shall revise the CQC plan reflecting the organizational changes and submit to the CITY for acceptance.

1.07 QUALITY CONTROL TESTING AND INSPECTION

- The CONTRACTOR shall perform inspections, tests, and other services as required by the Contract Documents, the approved CQC Program, and in accordance with laws, codes, rules, and regulations and document the results on checklists as described herein.

B.  CONTRACTOR shall provide written notice three (3) days prior to any on Site tests so that   the CITY may witness the CONTRACTOR and/or Subcontractors on Site tests. The CITY’s witnessing of tests does not relieve the CONTRACTOR and/or Subcontractors of their obligation to comply with the requirements of the Contract Documents.

- The CONTRACTOR shall develop an inspection and test schedule identifying all required inspections and tests as indicated in the Contract Documents and the approved CQC Program. The CONTRACTOR shall provide one copy of test and inspection results to the CITY’s representative witnessing the test, and retain, file and turnover the original inspection and test results in accordance with Section 01 45 29 Testing Laboratory Services.

D.   The CONTRACTOR shall maintain control over construction and installation processes to assure compliance with specified requirements. In-process and final inspection and testing of the Work shall be performed in accordance with written quality control, testing, and inspection procedures to assure that the requirements of the Contract Documents have been met.

E. The results of all inspections shall be recorded on inspection checklists developed by the CONTRACTOR and approved in writing by the CITY.

1.08 SUBMITTALS

- Submittals, if needed, shall be made as specified in section 01 33 00.

- The CONTRACTOR is responsible for certifying that all submittals and deliverables are in compliance with the contract requirements.

PART 2 – PRODUCTS

2.01	DELIVERABLES

A.  CQC Plan

B.  All quality records that are to be developed and turned over to the CITY in accordance with Specification Section 01 77 00 Contract Closeout.

PART 3 – EXECUTION

3.01	CONTRACTOR QUALITY CONTROL PLAN

- A required MSD CQC Plan template is available at Capital Project Resource Library (msdgc.org)

- The CONTRACTOR shall develop and maintain a CONTRACTOR’S Quality Control (CQC) Plan that is responsive to the requirements of this Specification Section, and includes procedures necessary to assure compliance with the requirements of the Contract Documents.

- The CQC Plan and implementing procedures for quality control of Work, including but not limited to each identified DFOW,  shall be submitted to the CITY for written approval prior to commencement of the Work but no later than thirty (30) days following receipt of the Notice to Proceed, whichever occurs first.

- The CQC Program shall explain the CONTRACTOR’S approach to on-site quality control, off-site quality control, the CQC organization, documentation of the CONTRACTOR and its Subcontractors quality control activities, and provide all other information necessary to demonstrate to the CITY that the CONTRACTOR, its Subcontractors, vendors and suppliers will provide quality control services that ensure compliance of the Work with the Contract Documents.

- The CQC Plan shall include, as a minimum, the following to cover all construction operations, both on-site and offsite, including work by subcontractors, fabricators, suppliers and purchasing agents:

- Introduction

  - Background

  - Purpose of CQC Plan

- Project Organization, Responsibility, and Points of Contact

- Description of quality control organization

- Organization Chart

- Resumes/Certifications/Documentation

- Quality Control Manager Delegation of Authority Letter

        - List of responsibilities

        - Stop work authorization

- List of Definable Features of Work

- Submittals

- Testing

- Control, verification, and acceptance testing procedures for each specific test to include the test name, specification paragraph requiring test, feature of work to be tested, test frequency, and person responsible for each test.

- Testing plan and log

- Inspection

- Procedures for tracking and recording preparatory, initial, and follow-up phases

- Documentation

- Nonconformance

- Procedures for tracking construction deficiencies

- Quality Control Forms

- Testing

- Contractor Quality Control Report

- Receiving Material Inspection Report

- Preparatory Meeting Checklist

- Initial Inspection Checklist

- Contractor’s Quality Control Report

- Construction Punch List

- Non-Conformance Report

- Weekly QA/QC Meeting Minutes

- Material Stamps

3.02	WEEKLY QUALITY CONTROL REPORT

A.  The CONTRACTOR shall deliver a weekly quality control report to the CITY. This report shall address:

1. Status of the CQC Program, including procedure development.

2. Contractor’s Daily Quality Control Reports

3. The number of inspections/tests performed during the week and results.

4. Any quality problems experienced.

5. Testing agency schedule updates.

6.  Non-Conformance Reports

7. Construction Punch List

8. Weekly QA/QC Meeting Minutes

9. Preparatory Meeting/Initial Inspection Checklists

10. Receiving Material inspection Reports

END OF SECTION 01 45 16